Exercise 1.

Explanation:

Read the given image,decrease the resolution as mentioned and observe the results

Procedure:

- Read the image using fopen() and fread() with the given length and width.

- As the image is 90 degree left rotated,so we rotated it 270 degree.

- Now for 2 pixels, we used imfilter().

- For 4 , 8 , 16 pixels,continued step 3.

- Observe the result

Code:

fileId = fopen('D:\Documents\DIP Lab\crossroad.dat','r');

a = fread(fileId,[580 435],'*uchar');

b = imrotate(a, 270);

figure, subplot(2,4,1), imshow(b), title('Original image');

i1 = imfilter(b, ones(2,2)/4);

i2 = imfilter(b, ones(4,4)/16);

i3 = imfilter(b, ones(8,8)/64);

i4 = imfilter(b, ones(16,16)/256);

subplot(2,4,5), imshow(i1), title('2x2');

subplot(2,4,6), imshow(i2), title('4x4');

subplot(2,4,7), imshow(i3), title('8x8');

subplot(2,4,8), imshow(i4), title('16x16');

figure, subplot(2,4,1), plot(b), title('Original image');

subplot(2,4,5), plot(i1), title('2x2');

subplot(2,4,6), plot(i2), title('4x4');

subplot(2,4,7), plot(i3), title('8x8');

subplot(2,4,8), plot(i4), title('16x16');

Output:

Observation:

- As image quality highly depends on resolution, it is not same for all images.

- It differs in different portion and in different object in same image.

- In the given image, the tall tree and the car fade faster than other objects.

Exercise 2.

Problem Statement:

Explanation:

The problem is to read the given image and gradually reduce the number of bits to represent each pixel and observe the result.

Procedure:

- Read the image using fopen() and fread()  with given line length.

- As the given is rotated 90 degree left, we  rotate it 270 degree.

- Make a new image with same length and width of the original image.

- Assign each pixel with original image by diving it by 8 for 5 bits pixel.

- Follow step 4 by diving the original image by 16, 32, 64 and 128.

- Observe the output.

Code:

fileId = fopen('D:\Documents\DIP Lab\crossroad.dat','r');

a = fread(fileId,[580 435], '*uint8');fclose all;

b = imrotate(a, 270);

b = uint8(b);

i1  = b./8;

i2  = b./16;

i3  = b./32;

i4  = b./64;

i5  = b./128;

figure,

subplot(2,3,1), imshow(b), title('Original');

subplot(2,3,2), imshow(mat2gray(i1)), title('5 bit');

subplot(2,3,3), imshow(mat2gray(i2)), title('4 bit');

subplot(2,3,4), imshow(mat2gray(i3)), title('3 bit');

subplot(2,3,5), imshow(mat2gray(i4)), title('2 bit');

subplot(2,3,6), imshow(mat2gray(i5)), title('1 bit');

Output:

Observation:

The coloring details decays as the number of bits reduces and finally it becomes a fully binary image.

Exercise 3.

Explanation:

The problem is to read the given image. From grayscale image, we have to change it to a blue image. Then we have to change the color gradually from black to green to yellow to white for given range of pixel value.

Procedure:

- Read the image using fopen() and fread() with given line length.

- Make a new image with same height and width and depth of 3.

- Assign the given image to third depth of new image which is the blue layer.

- Make another new image with same height and width and depth of 3.

- For pixel value 0 to 100, assign the pixel value of the image to the green layer.

- For pixel value 101 to 200, assign the pixel value of the image to green layer and red layer to make the image yellowish.

- For pixel value 201 to 255, assign the pixel value of the image to green , red and blue layer to whiten the image.

- Observe the output.

Code:

fileId = fopen('D:\Documents\DIP Lab\standford.dat', 'r');

a = fread(fileId, [580 435], '*uint8');

b = imrotate(a, 270);

imgblue = zeros(435, 580, 3);

imgblue(:, :, 3) = mat2gray(b);

imgnew = uint8(zeros(435, 580, 3));

for i = 1: 435

for j = 1: 580

if b(i, j) <= 100

imgnew(i, j, 1) = 0;

imgnew(i, j, 2) = 255;

imgnew(i, j, 3) = 0;

elseif b(i, j) <= 200

imgnew(i, j, 1) = 255;

imgnew(i, j, 2) = 255;

imgnew(i, j, 3) = 0;

else

imgnew(i, j, 1) = 255;

imgnew(i, j, 2) = 255;

imgnew(i, j, 3) = 255;

end

end

end

figure, subplot(1,3,1), imshow(b), title('Original Image');

subplot(1,3,2), imshow(imgblue), title('Blue version');

subplot(1,3,3), imshow(imgnew), title('Other version');

Output:

Exercise 4.

Explanation:

The problem is to read an RGB image. Then change it to HSV image. After that, rotate the hue 120 degree and -120 degree. And get the same result processing an RGB image.

Procedure:

- Read the image using fopen and fread() with given line length.

- Change it to HSV image using rgb2hsv() function.

- Take the hue layer of HSV image and rotate the value 120 degree in both directions.

- Revert it to RGB image using hsv2rgb() function and display.

- Rotating 120 degree means replacing red by blue, green by red and blue by green in RGB image. This also can be achieved by swapping color planes.

- Rotating -120 degree means replacing red by green, green by blue and blue by red in RGB image. We can achieve it by swapping the color planes.

Code:

i = imread('D:\Documents\DIP Lab\flip.jpg');

hsvImg = rgb2hsv(i);

hue = hsvImg(:, :, 1) * 360;

hsvImg(:,:,1) = (mod(hue + 120, 360)) / 360;

img120 = hsv2rgb(hsvImg);

hsvImg(:,:,1) = (mod(hue - 120, 360)) / 360;

imgneg120 = hsv2rgb(hsvImg);

[h, w, d] = size(i);

rgb120 = uint8(zeros(h, w, d));

rgb120(:, :, 1) = i(:, :, 3);

rgb120(:, :, 2) = i(:, :, 1);

rgb120(:, :, 3) = i(:, :, 2);

rgbneg120 = uint8(zeros(h, w, d));

rgbneg120(:, :, 1) = i(:, :, 2);

rgbneg120(:, :, 2) = i(:, :, 3);

rgbneg120(:, :, 3) = i(:, :, 1);

subplot 231; imshow(i);title('Original');

subplot 232;imshow(img120);title('120 Degree Rotated HSV');

subplot 233; imshow(imgneg120), title('-120 Degree Rotated HSV');

subplot 235;imshow(rgb120), title('120 Degree Rotated RGB');

subplot 236;imshow(rgbneg120), title('-120 Degree Rotated RGB')

Output: